Introduction of RTOS

RTOS (Real-Time Operating System) is a special operating system designed for embedded systems that must respond to events within a guaranteed time. Unlike a general-purpose operating system (such as Windows or Linux), an RTOS focuses on predictable timing and reliable task execution.

### Why RTOS is Used in Embedded Systems

RTOS is used when timing is critical, such as in:

- Automotive systems (ABS, airbags)

- Medical devices (heart monitors, infusion pumps)

- Industrial automation

- Robotics

- Consumer electronics (smart TVs, cameras)

- IoT devices

Example

### Car Airbag System

Tasks:

- Monitor crash sensors.

- Calculate impact severity.

- Deploy airbags.

- Monitor Crash Sensors

  - Sensors (accelerometers and impact sensors) continuously measure vehicle acceleration and deceleration.

  - The microcontroller reads sensor data every few milliseconds.

- Calculate Impact Severity

  - The processor analyses the sensor data.

  - It determines whether the impact is severe enough to require airbag deployment.

- Deploy Airbags

  - If the impact exceeds a predefined threshold, the controller sends a signal to the inflator circuit.

  - The inflator rapidly fills the airbag with gas.

| Task | Priority |
| --- | --- |

| Crash Detection | Highest |
| --- | --- |

| Airbag Deployment | Highest |
| --- | --- |

| Dashboard Display | Medium |
| --- | --- |

| Radio Control | Low |
| --- | --- |

When a collision is detected:

- The RTOS immediately switches the CPU to the crash-detection task.

- Lower-priority tasks are paused.

- The impact is analysed.

- The airbag deployment task runs immediately.

Another example

### Traffic Light Control System

Tasks:

- Control Red, Yellow, and Green lights.

- Detect emergency vehicles.

- Update countdown display.

### Without RTOS

You would typically use:

while(1)

{

Read_ADC();

Send_UART();

Toggle_LED();

}

### With RTOS

Each function runs in its own task:

Task_ADC();

Task_UART();

Task_LED();

The RTOS scheduler decides which task runs and when, based on priority and timing requirements.

Normal OS (Windows):

Open Browser → Open Word → Play Music

The exact timing is not critical.

RTOS (Airbag Controller):

Detect Crash → Analyse Impact → Deploy Airbag

Timing is critical and deadlines must be met.

So, normal operating systems are used for everyday computing tasks where occasional delays are acceptable, while RTOS is used when timing is critical.

Difference between RTOS and Normal Operating System

| Feature | RTOS (Real-Time OS). | Normal OS (Windows, Linux, Android) |
| --- | --- | --- |
| Main Goal | Meet timing deadlines | Maximize user experience and throughput |
| Response Time | Predictable and guaranteed | Not guaranteed |
| Task Scheduling | Priority-based | Fairness-based |
| Deadline Miss | Usually unacceptable. | Generally acceptable |

### Popular RTOS Examples

- FreeRTOS

- Zephyr

- VxWorks

- ThreadX (Azure RTOS)

- QNX

- RTEMS

### Types of Real-Time Operating System

The real-time operating systems can be of 3 types:

Hard real-time system

hard real-time system, missing a deadline is considered a system failure.

#### 1. Airbag Control System (Automobile)

- Detect collision.

- Analyse crash severity.

- Deploy airbag within milliseconds.

- Missing the deadline can cause serious injury

Soft Real-Time System

In a soft real-time system, deadlines are important, but occasionally missing a deadline does not cause system failure.

### Audio Player

- Read audio data.

- Process sound.

- Send data to speakers.

A small delay may cause a brief audio disturbance, but it is not dangerous.

### Firm Real-Time Operating System (Firm RTOS)

A Firm Real-Time System lies between Hard Real-Time and Soft Real-Time systems.

- Deadlines are important.

- Missing a deadline does not cause system failure.

- However, the result becomes useless if the deadline is missed.

### Example

#### Radar Tracking System

- Detect aircraft position.

- Process radar data every 100 ms.

- Update tracking display.

If one update arrives after 100 ms:

- The system continues running.

- The late data is discarded because it is no longer useful.

Application: Two tasks running under FreeRTOS

- Task 1: Blink LED every 500 ms

- Task 2: Send a message over UART every 1 second

### IOC Configuration

- Enable PG13 as GPIO_Output (LED).

- Enable USART3 (Asynchronous mode).

- Middleware → FreeRTOS → CMSIS_V2.

- Create two tasks:

  - LED_Task

  - UART_Task

#include "main.h" #include "cmsis_os.h" #include <string.h> extern UART_HandleTypeDef huart3; /* Task handles */ osThreadId_t LEDTaskHandle; osThreadId_t UARTTaskHandle; /* LED Task */ void StartLEDTask(void *argument) { for(;;) { HAL_GPIO_TogglePin(GPIOG, GPIO_PIN_13); osDelay(500); } } /* UART Task */ void StartUARTTask(void *argument) { char msg[] = "FreeRTOS Running\r\n"; for(;;) { HAL_UART_Transmit(&huart3, (uint8_t *)msg, strlen(msg), HAL_MAX_DELAY); osDelay(1000); } }

Execution how

osKernelInitialize() ¦ v osThreadNew(StartTask02, NULL, &ledtask1_attributes) ¦ v Task Created ¦ v ledtask1Handle stores task handle ¦ v osKernelStart() ¦ v StartTask02() begins running

Inter-Task Communication in RTOS

Inter-Task Communication (ITC) is the method used by one RTOS task to exchange data or signals with another task.

### Why is it needed?

Suppose:

- Task 1 reads a sensor.

- Task 2 displays the sensor value on UART.

Task 1 must send the sensor data to Task 2.

Sensor Task ---> UART Task 25°C

This data transfer is called inter-task communication.

Inter-Task Communication Methods

  - Queue

A queue stores data and transfers it safely between tasks.

Task 1 Queue Task 2 ------- ------- ------- Read ADC ----100----> [100] ----100----> Display

Example :

Task1 (Sender Task) → Sends numbers to Queue

Task2 (Receiver Task) → Receives numbers and prints through UART

ioc configuration

Middleware → FreeRTOS

Create queue

| Parameter | Value |
| --- | --- |
| Name | myQueue |
| Queue Size | 5 |
| Item Size | 4 |

| Create task Task1 Parameter; Value.; Parameter; Value.. Name.; SenderTask.. Priority.; Normal.. Stack Size.; 128.. |  |
| --- | --- |
| Task2 Parameter; Value.; Parameter; Value.. Name.; ReceiverTask.. Priority.; Normal.. Stack Size.; 128.. Task1 void StartSenderTask(void *argument) { uint32_t txData = 0; for(;;) { txData++; osMessageQueuePut(myQueueHandle, &txData, 0, osWaitForever); osDelay(1000); } }. |  |

Task2

extern UART_HandleTypeDef huart2; void StartReceiverTask(void *argument) { uint32_t rxData; char msg[50]; for(;;) { if(osMessageQueueGet(myQueueHandle, &rxData, NULL, osWaitForever) == osOK) { int len = sprintf(msg, "Received Data = %lu\r\n", rxData); HAL_UART_Transmit(&huart2, (uint8_t*)msg, len, HAL_MAX_DELAY); } } }

Mutex

A Mutex (Mutual Exclusion) is a synchronization mechanism used to protect shared resources from being accessed by multiple tasks at the same time.

Why Mutex is Needed

Suppose two tasks want to use the same UART:

- Task 1 sends "Hello"

- Task 2 sends "World"

Without a mutex, the output may become:

HeWlolrold

because both tasks access the UART simultaneously.

Working of Mutex

Task 1 ----> Take Mutex ----> Use Resource ----> Give Mutex Task 2 ----> Wait ---------> Take Mutex -----> Use Resource

A mutex allows only one task at a time to access the UART.

Task 1 → [TAKE MUTEX] → UART → [RELEASE MUTEX] ¦ Task 2 → waits -----------------------+ ¦ [TAKE MUTEX] → UART → [RELEASE]

Example:

Task01 Name: Task01 Priority: Normal Stack Size: 128 Task02 Name: Task02 Priority: Normal Stack Size: 128

### Create Mutex

Go to:

FreeRTOS → Mutexes

Task1

void StartTask01(void *argument) { for(;;) { osMutexAcquire(UART_MutexHandle, osWaitForever); HAL_UART_Transmit(&huart2, (uint8_t *)"Task 1\r\n", 8, HAL_MAX_DELAY); osMutexRelease(UART_MutexHandle); osDelay(1000); } }

Task2

void StartTask02(void *argument) { for(;;) { osMutexAcquire(UART_MutexHandle, osWaitForever); HAL_UART_Transmit(&huart2, (uint8_t *)"Task 2\r\n", 8, HAL_MAX_DELAY); osMutexRelease(UART_MutexHandle); osDelay(1000); } }